This document applies to command stations from:

North Coast Engineering (Webster, NY)

Wangrow Electronics Inc. (Park Ridge, IL)

Ramtraxx (Toronto),  PSI Dynatrol (Boston)

# 1	Overview of Controller Bus

The controller bus connects up to 63 Hand-Held Cab Controllers to the Command Station.  Data on the bus runs at 9600 bps with 8 data bits, no parity, 2 stop bits and meets RS-485 specifications.  Controllers are continually in receive unless specifically addressed by the command station at which time the command station relinquishes the bus to allow the controller to respond.   All communications sent from the command station to the Hand-Held Controllers must have bit 7 set.   A "ping" has bit 6 reset to 0 and all other commands or data have bit 6 set.  All communications from the cab controllers to the command station have bit 7 reset.   The maximum length of the bus is 500 feet and should be a continuous daisy chain without "branches" of any significant length (longer than 8 feet).

An RJ-11 is the recommended "basic" connector for the bus for cost reasons.   Only the center 4 wires of the bus are normally used.  A 5 pin 180 degree DIN connector is also acceptable especially when wired to be compatible with the Lenz pinout.   In addition it is possible to easily build an adapter that converts the 4 wire bus to a 3 wire bus to enable the use of standard 1/4" phone plugs.

Wiring is as follows:

| PIN 1 | +12 Volts (nominal) |
| --- | --- |
| PIN 2 | Ground |
| PIN 3 | "A" lead of RS-485 signal |
| PIN 4 | "B" lead of RS-485 signal |

3 wire plug wiring:

| Tip | +12 Volts (7.5 to 16 Volts DC is OK) |
| --- | --- |
| Ring | RS-485 "A" lead |
| Sleeve | Ground |

RJ-11 Wiring:

Pin 1 is on the right when looking into the end of jack with the wires at the top of the jack

| PIN 1 | no connect |
| --- | --- |
| PIN 2 | +12 Volts DC |
| PIN 3 | "A" (non-inverted) lead of RS-485 (to pin 6 of 75176 chip) |
| PIN 4 | "B" (inverted) lead of RS-485 (to pin 7 of 75176 chip) |
| PIN 5 | Ground |
| PIN 6 | open collector RS-485 "direction" wire from command station low = data from command station, high = data to command station |

In normal operation up to sixty-three controllers are polled (pinged) for changes of status.  A cab, when addressed, will answer with a two byte response indicating button press (1st byte) and speed (2nd byte).  The cab starts sending it's response approximately 750 mSec. from the receipt of the last stop bit from the command station.   Cab 0 is the "broadcast" address.  Cab #1 is reserved for future applications.

# 2	Protocol For Operation Of Controller Bus

## 3	Normal Bus Operation

- Base "pings" an address.

- Controller answers with a 2 byte response, the first byte is a key press response, the second byte is speed.   If no  controller response, jump to step 1.  It is not necessary for a cab to answer if there is nothing to send.

- If base has any commands or data for the controller it will send the command/data immediately after receiving a ping response from the controller.  Data will start within 300us of last stop bit from cab.

- Jump back to step 1 to ping next address.

## 4	Power Up Operation of Base Station

- Base sends nothing for 2 seconds to allow Hand-Held controllers to 'settle down' (and user time to read address, other information on LCD)

- Base "pings" an address.

- If a response is received correctly and DIP switch 7 is open the base sends: "CMD STA  00:00AM", "THU DEC 29 1994".  (or other Command Station software version) for display on the LCD.  If DIP switch 7 is closed then the command station will send: "LOC:1403  05:34PM", "FWD: 000 L--3----" (or whatever the last display was for this cab).

- Start normal operations.

## 5	Power Up Operation of Controller

- Clear LCD display.  If the "SELECT LOCO" button is pushed during power up the controller will go into it's internal setup mode.   This is how the cab address is set.

- Wait for ping to cab number 0.  By waiting to hear a ping for cab 0 you will be able to easily "sync up" with the cab bus.

- Wait for ping this cab, then request last LCD "screen" from command station.  If this is a total system power up for the system the command station will ignore the request.  The command station keeps an image of each cab's LCD screen in memory so the cab LCD can be refreshed as the cab is transferred from location to location on the cab bus.

- Respond to commands from the command station.

- If an "Enter controller setup mode" command is received the cab displays it's current address for modification.

- Goto step 1 of normal bus operation.

# 6	Description of Command Station Transmissions

NOTE: all transmissions from command station must begin with the most significant bit set to 1

## 7 Ping (1 byte, but a multi-byte command may follow after cab answers)

10 <address>	6 bit address 1-63 are normal cab addresses, 0 is the broadcast address.

A ping is the only transmission from the command station with bit 6 cleared to 0.

## 8 Commands (multi-byte)

All bytes from base to a cab must have bits 6 and 7 of a multi-byte command set to 1.   Only ASCII characters in the range of 20h-5Fh may be printed.  The command station sets bit 6 if bit 5 is set before transmitting it, the cab is expected to adjust the bits back [ie. if bit 5=1 then clear bit 6, if bit 5=0 don’t clear bit 6]  The LCD display of type A cabs is assumed to be a 2x16 or 4x16 character display.

| 11000000 | print next 8 chars to 1st line left LCD (addr = 80h) |
| --- | --- |
| 11000001 | print next 8 chars to 1st line right of LCD (addr = 88h) Note: this is where the clock prints |
| 11000010 | print next 8 chars to 2nd line left of LCD (addr = C0h) |
| 11000011 | print next 8 chars to 2nd line right of LCD (addr = C8h) |
| 11000100 | print next 8 chars to 3rd line left of LCD (addr = 90h) |
| 11000101 | print next 8 chars to 3rd line right of LCD (addr= 98h) |
| 11000110 | print next 8 chars to 4th line left of LCD (addr = D0h) |
| 11000111 | print next 8 chars to 4th line right of LCD (addr = D8h) |
| 11001000 | move cursor to LCD address in following byte <C8h> <80h> move cursor to address 80 (home) |
| 11001001 | print char TTY fashion to LCD, back up cursor to same spot <C9h> <B9h> print "9" and return cursor to same position |
| 11001010 | print char TTY fashion to LCD, move cursor to next location <CAh><B9h> print "9" to LCD, let cursor advance to next spot |
| 11001011 | upload 1st following byte graphic location with next 8 bytes. Bits 5,6,7 must be reset in the controller. Ex: program graphic location 00 with a vertical line. Note: the graphic char value (0-7) is last. <CBh><D0h><D0h><D0h><D0h><D0h><D0h><D0h><D0><C0h> |
| 11001100 | print graphic char indicated by following byte (bits 6,7 are set in the second byte to comply with bits 7 and 6 must be set rule) <CCh><C3h> |
| 11001101 | clear display and home cursor <CDh> |
| 11001110 | cursor off <CEh> (LED OFF in basic cabs) |
| 11001111 | cursor on <CFh> (LED ON in basic cabs) |
| 11010000 | shift LCD to display the 'right half' of the screen on a two line display. <D0h> |
| 11010001 | non-destructive home screen back to 'left half' <D1h> |
| 11010010 | cab returns a one byte “cab type” starting at 60h ('a' for cabs with LCD displays, 'b' for cabs without display capabilities, type 'c' is reserved, 'd' is for Auxillary Input Units, ‘e’ is for Cab-Bus/X-bus Bridge Units, ‘f’ for Cab Bus/Loconet bridge unit. |
| 11010011 | cab enters it's own setup mode (to set it's address) <D3h> |
| 11010100 | light up home green cab signal on cab (clears yellow and red) |
| 11010101 | light up home yellow cab signal on cab (clears green and red) |
| 11010110 | light up home red cab signal on cab (clears yellow and green) |
| 11010111 | light up away green cab signal on cab |
| 11011000 | light up away yellow cab signal on cab |
| 11011001 | light up away red cab signal on cab |
| 11011010 | Cab is to return 2 byte address of loco to control (response to cab key code 6Eh) If long address - cab returns NMRA style extended address (ie 11aaaaaa aaaaaaaa) If short address - cab returns first byte of 00h then short address (ie 00000000 0aaaaaaaa) |
| 11011011 | 4 bytes following this byte contain information about the currently controlled locomotive Byte 1 <DBh> (4 bytes of loco info follow) Byte 2 bit 5 = direction (1=forward, 0=reverse) - bit 4 = \ - bit 3 = \ - bit 2 = > 5 most sig. bits of current speed - bit 1 = / - bit 0 = / Byte 3 bit 5 = \ 2 least sig. bits of current speed - bit 4 = / - bit 3 = \ - bit 2 = \ 4 most sig. bits of loco addr high byte. bit 2 (bit 6 of addr hi byte) =1 if in - bit 1 = / 128 speed mode, bit 2=0 if 14/28 - bit 0 = / Byte 4 bit 5 = \ - bit 4 = \ - bit 3 = / 4 least sig. bits of loco address high byte - bit 2 = / - bit 1 = \ - bit 0 = / 2 most sig. bits of loco address low byte Byte 5 bit 5 = \ - bit 4 = \ - bit 3 = \ 6 least sig bits of loco address low byte - bit 2 = / - bit 1 = / - bit 0 = / |
| - 11011100 | Activate buzzer or other attention getting device on cab (“Beep” operator) |
| 11011101 through |  |
| 11111111 | reserved for factory test modes |

NOTE: commands C0h-CDh, D0h and D1h ignored by cabs which have no LCD display.

# 9	Description of Hand-Held Controller Transmissions

NOTE: All transmissions from Hand-Held controller begin with 0

## 10 Ping Response

FIRST BYTE (key press or other information.   NOTE: 7Fh never sent in 1st byte):

Type a, b and c cabs:

| 010xxxxx | Key press info (40h-7Ch) |
| --- | --- |
| 01111101 | No key to report <7Dh> |
| 01111110 | Repeat current screen <7Eh> |

Type d cabs:

| 0xxxxxxx | Type 'd' auxillary input units bit map for inputs 1-7, bit 0 is input 1, bit 6 is input 7. |
| --- | --- |

SECOND BYTE (speed when using knob):

Type a, b and c cabs:

| 0xxxxxxx | Speed control from knob (range 0-126, 127 = knob not used) |
| --- | --- |
| 01111111 | No speed to report, keys are being used <7Fh> |

Type d cabs:

| 0xxxxxxx | Type 'd' auxillary input units bit map for inputs 8-14, bit 0 is input 8, bit 6 is input 14 |
| --- | --- |

KEY CODES:

| 40 - Enter | 60 - Assign Cab -> loco |
| --- | --- |
| 41 - "Program" Key | 61 - Program on Mainline |
| 42 - Recall | 62 - Set Clock |
| 43 - Direction Toggle | 63 - Program on Setup track |
| 44 - Setup Consist | 64 - Setup command station |
| 45 - Add Loco to Consist | 65 - Setup cab (cab enters its own setup mode) |
| 46 - Delete Loco from Consist | 66 - Setup macros |
| 47 - Kill Consist | 67 - Setup brute force consist |
| 48 - Select Loco (by address) | 68 - Setup advanced consist |
| 49 - Horn/Whistle key down | 69 - Toggle LCD between left/right half |
| 4A - 1 step faster | 6A - Set direction to forward |
| 4B - 1 step slower | 6B - Set direction to reverse |
| 4C - Emergency stop | 6C - "key 32" -unused at this time |
| 4D - Bell | 6D - "key 33" -unused at this time |
| 4E - Enter Accessory mode | 6E - new loco address ready (initiates multibyte command 0xDA) |
| 4F - Expansion key | 6F - F0 off, F1 off (simultaneous control of 2 functions) |
| 50 - Headlight Toggle/0 | 70 - F0 on, F1 off (simultaneous control of 2 functions) |
| 51 - F1 Toggle/1 | 71 - F0 off, F1 on (simultaneous control of 2 functions) |
| 52 - F2 Toggle/2 | 72 - F0 on, F1 on (simultaneous control of 2 functions) |
| 53 - F3 Toggle/3 | 73 - all functions off, F0-F8 |
| 54 - F4 Toggle/4 | 74 - “Alt” key (currently undefined) |
| 55 - F5 Toggle/5 | 75 - reserved |
| 56 - F6 Toggle/6 | 76 - reserved |
| 57 - F7 Toggle/7 | 77 - reserved |
| 58 - F8 Toggle/8 | 78 - reserved |
| 59 - 9 (type 'b' direction) | 79 - reserved |
| 5A - 5 steps faster | 7A - reserved for factory test mode |
| 5B - 5 steps slower | 7B - reserved for factory test mode |
| 5C - Macro select | 7C - Swap current cab context page of this command station with another command station via RS-232 port (expansion to 120 cabs using 2 command stations). Effectively swaps this cab out to another command station. |
| 5D - 14/28 speed toggle | 7D - no key pressed |
| 5E - Brake (set speed to zero) | 7E - repeat last LCD display (also requests info on current loco) |
| 5F - Horn/Whistle key up | 7F - Using keys for speed control |

# 11	Timing Of Cab Bus

At 9600,N,8,2 each byte takes 1.144 mS

HARD AND FAST RULE:  	Hand-Held Controller must start to respond  within 900 uS to a ping 					or not respond at all.

Therefore a complete round of 64 pings with simple responses will take:

( 1.144 mS + .75 mS + 2.288 mS +  .1 mS ) * 64pings = 274 mS

|	|	     |	      |

|	|	     |	      |---	delay to start of next ping

|	|	     |---------------	normal length of cab response (2 bytes)

|	|------------------------------	normal delay to start of cab response

|----------------------------------------	base ping (length of 1 byte at 9600bps)

NOTE:  If a cab does not start to respond within 900uS the second byte is not waited for and the command station will then ping the next cab.  This has the effect of speeding up the ping process to remaining cabs.

Starting with command station software dated 7/29/95 and later:

- Cabs are not necessarily pinged in numerical order.

- If a cab is off the bus for several pings the command station will drop the cab from it's "ping list".  That cab will then only be pinged on an occasional basis until it again responds to a ping.  This  dynamic ping list replaces the DIP switch setting of the maximum number of cabs inside the command station.  It also dramatically  speeds up cab response when more than 30 cabs are in use.